Cyber Security Framework Planning and Establishment of Operational Technology Security Operations Centres (OT-SOCs) for Indian Railways

An architecture-driven methodology integrating IEC 62443, EN 50159, TS 50701 and Kavach cyber security requirements with field-proven OT-SOC foundational practice

Prepared for the IRSE India Technical Convention  |  Audience: IRSSE Officers, RDSO, Railway Board, Signalling Engineers and OT Cyber Security Practitioners

# Abstract

The nationwide deployment of the Train Collision Avoidance System (Kavach), electronic interlocking (EI), centralised Traffic Management Systems (TMS) and converged Multi-Protocol Label Switching (MPLS) telecom backbones has transformed Indian Railways signalling from a collection of electrically isolated vital circuits into a networked, safety-critical cyber-physical system. This convergence exposes Operational Technology (OT) to a threat surface that enterprise Information Technology (IT) Security Operations Centres (SOCs) are neither designed nor competent to monitor. This paper presents a structured, architecture-driven methodology for planning and establishing dedicated Operational Technology Security Operations Centres (OT-SOCs) for Indian Railways. Its central thesis is that a machine-validated, standards-traceable model of the signalling architecture - its IEC 62443 zones and conduits, its EN 50159 transmission categories and its Safety Integrity Level (SIL) allocations - must precede and continuously inform SOC operating-model selection, sensor placement, detection engineering, incident response and governance. The evidence base is an independent cyber security assessment of a reference railway OT architecture model in which the semantic, governance and enforcement authorities were formally separated and validated; the assessment surfaced durable engineering findings (absence of cryptographic authentication on vital Category-1 buses, absence of multi-factor authentication on privileged engineering access, absence of timeliness monitoring on open radio bearers, incompletely instrumented trust boundaries, and an unprotected telecom backhaul) together with standards-compliance findings (the EN 50159 / IEC 62443 authentication-crediting tension and the need for TS 50701 lifecycle evidence). The paper integrates this railway-specific evidence with field-proven OT-SOC foundational practice: an evaluation of five OT-SOC operating models with a recommendation for Indian Railways; a five-layer visibility architecture applied to Electronic Interlocking, Kavach, MPLS, radio networks, TMS, NMS, data loggers and engineering workstations; the Monitoring Paradox and its grounding in SIL-4 signalling, IEC 62443 and EN 50159; a four-gate, safety-first incident-response doctrine; a railway SEV1-SEV4 severity model; a three-tier (Board / SOC-Lead / Analyst) KPI framework; a railway OT-SOC staffing and competency model; a national governance model spanning Railway Board, RDSO, NCIIPC, CERT-In, Zonal Railways and Divisions; a residual-risk methodology with explicit risk-acceptance principles for SIL-4 systems; a five-level OT-SOC maturity roadmap; and a 90-day pilot-deployment programme. The paper is offered as publishable planning guidance for the establishment of OT-SOC capability across Indian Railways.

Keywords: OT-SOC; Indian Railways; Kavach; IEC 62443; EN 50159; TS 50701; Electronic Interlocking; Zones and Conduits; Detection Engineering; SIL-4; Incident Response; Monitoring Paradox; Residual Risk; SOC Maturity; NCIIPC; CERT-In; RDSO.

# 1. Introduction

For most of its history, the safety of railway signalling rested upon two principles: physical isolation and fail-safe design. Vital functions were realised in relay logic, hard-wired track circuits and electrically isolated lineside cabling; the cyber threat surface was, in practice, the perimeter fence. The doctrine of security through isolation was tenable because the signalling system genuinely was isolated.

That premise no longer holds. The contemporary Indian Railways signalling estate is a converged, IP-enabled, geographically distributed cyber-physical system. Electronic interlockings exchange vital state with object controllers over digital buses; the Kavach Automatic Train Protection (ATP) system extends vital communication across an open radio-frequency (RF) air interface between the Stationary Kavach (S-Kavach) at the wayside and the Loco Kavach (L-Kavach) onboard; TMS and Network Management Systems (NMS) supervise wide geographies over MPLS backbones; and engineering workstations capable of downloading vital logic connect, through jump hosts and remote-access infrastructure, into the heart of the safety domain. Each of these capabilities is a deliberate engineering advance; each also introduces a conduit through which a determined adversary may attempt to influence a safety function.

The instinctive institutional response - to extend the existing enterprise IT SOC to also cover signalling - is inadequate and, in places, dangerous. Cross-sector OT-SOC experience is unambiguous: extending an IT SOC into OT consistently fails because the tools, telemetry sources, threat models, response playbooks and human expertise required are fundamentally different, producing a false sense of security - monitoring coverage that looks credible on paper but is blind to the threats that matter most. An IT SOC optimises for confidentiality and the protection of data; an OT-SOC must optimise for the preservation of safety and the availability of a physical process. An IT SOC may legitimately quarantine an endpoint or block a flow on detection; an OT-SOC that interrupts a vital communication path may itself precipitate the hazard it was deployed to prevent.

This paper argues that the establishment of OT-SOC capability for Indian Railways must be a discipline in its own right, and that its single most important precondition is an architecture that has been validated against standards before it is monitored. One cannot monitor what one has not modelled; one cannot prioritise detection on a flow whose criticality, transmission category and trust boundary are unknown; and one cannot defend a zone whose conduits have not been enumerated. The argument is developed from concrete evidence - an independent, standards-traceable assessment of a reference railway OT architecture model - and fused with field-proven OT-SOC foundational practice, so that Indian Railways need not re-derive at cost what other critical-infrastructure sectors have already learned.

# 2. Evolution of Cyber Security in Indian Railways

Four phases characterise the trajectory of railway signalling cyber security in India. Phase I - electromechanical isolation - comprised relay interlockings and token working; security was physical and procedural and no cyber threat surface of consequence existed. Phase II - first-generation electronics - introduced solid-state and electronic interlockings, data loggers and digital axle counters; digital logic entered the vital domain but communication remained local, proprietary and serial. Data loggers, installed for diagnostics, became almost incidentally the first forensic record of what would later be recognised as security events. Phase III - networked OT and the rise of the backbone - brought MPLS backbones, centralised TMS, NMS and remote diagnostics, the first genuine IT/OT convergence; engineering access, previously requiring physical presence in the relay room, became remotely brokered, and the threat surface expanded from the fence to the network.

Phase IV - safety over open bearers - is the present, and its defining event is the nationwide rollout of Kavach. Kavach deliberately carries vital movement-authority information across an open RF bearer between wayside and locomotive and integrates with the electronic interlocking at the station. For the first time, a vital, SIL-4 function depends upon communication over a transmission system that, in EN 50159 terms, is open - one in which the population of participants and the medium itself cannot be fully controlled. The RDSO Kavach specification and the associated Kavach Cyber Security Standard Operating Procedure (SOP), together with Indian Railways' signalling cyber security policy direction, recognise that cryptographic protection of the Kavach interface is necessary but not sufficient: protection without monitoring provides no assurance that the protection is functioning, no detection of attempts to defeat it, and no evidence for incident response. The OT-SOC is the institutional mechanism through which protection is continuously verified, and its establishment is therefore no longer optional but a regulatory and safety imperative.

# 3. The Railway OT Threat Landscape

The railway OT threat landscape differs from the enterprise landscape in its consequences, its adversaries and its constraints. Two decades of cross-sector incident response show that OT threats cluster into five operationally distinct categories; Table 1 maps these to their railway manifestations and to the consequence that matters in signalling - the wrong-side failure.

Table 1. Cross-sector OT threat categories mapped to Indian Railways

| # | Generic OT threat category | Indian Railways manifestation | Consequence |
| --- | --- | --- | --- |
| 1 | IT-sourced lateral movement (ransomware crossing a weak IT/OT boundary; Colonial Pipeline class) | Enterprise to IDMZ to operations to interlocking traversal where boundary controls are incompletely instantiated | Disruption of TMS/NMS; staging toward vital zones |
| 2 | Engineering-workstation compromise (TRITON/TRISIS modified safety logic, 2017) | Compromise of an engineering workstation with logic-download access to EI / S-Kavach | Alteration of vital interlocking logic - wrong-side failure |
| 3 | Supply-chain / vendor-access exploitation (SolarWinds scale) | Vendor laptop or software-update channel for Kavach / EI / TMS subsystems | Implant introduced into the vital supply chain |
| 4 | Insider threat and sabotage (Maroochy Water, 2000) | Maintainer or contractor with privileged engineering access and protocol knowledge | Setpoint/logic change; alarm suppression on a vital system |
| 5 | Nation-state pre-positioning and destructive attack (INDUSTROYER, TRITON, PIPEDREAM) | Persistent access to Kavach/EI/telecom infrastructure awaiting operational timing | Coordinated destructive disruption of national rail safety |

The worst-case outcome in railway OT is not data loss but a wrong-side failure: a signal cleared against a conflicting movement, a point moved under a train, a movement authority issued where none is safe, or the suppression of an emergency brake command. The safety-criticality of the affected asset - captured formally as its SIL - must therefore be the primary axis along which monitoring effort is prioritised. The relevant adversary spectrum runs from the opportunistic (commodity malware introduced via an engineering laptop or USB device) through the capable insider (a maintainer with privileged access and detailed protocol knowledge) to the resourced nation-state actor. The capable insider is of particular concern because the vital buses rely, by design and by standard, on closed-transmission assumptions and safety coding rather than cryptographic source authentication - a defence robust against the outsider but materially weaker against an actor who has obtained access to the closed segment. Critically, the majority of OT incidents that require SOC response begin with human error or supply-chain compromise, not sophisticated attack: a well-designed railway OT-SOC must be equally capable of detecting configuration drift, accidental connectivity changes and unauthorised removable media as of hunting nation-state implants.

# 4. Standards Framework: IEC 62443, EN 50159, TS 50701 and Kavach

Four standards and policy families provide the structural grammar of railway OT security, and the OT-SOC must be built upon all of them simultaneously.

## 4.1 IEC 62443 - Zones, conduits and security levels

IEC 62443-3-2 (zone and conduit risk assessment) and IEC 62443-3-3 (system security requirements and security levels) decompose a system into zones - groupings of assets sharing common security requirements - connected by conduits, the controlled communication paths between zones. Its seven Foundational Requirements - FR1 Identification and Authentication Control, FR2 Use Control, FR3 System Integrity, FR4 Data Confidentiality, FR5 Restricted Data Flow, FR6 Timely Response to Events, and FR7 Resource Availability - give a complete vocabulary for security controls. Crucially for the OT-SOC, FR6 is itself a monitoring mandate. The reference architecture realises the IEC 62443 model as an explicit zone hierarchy - Enterprise (L5), IDMZ and Security-Management (L3.5), Operations (L3), Telecom Core (L2), Radio Access (L1), Interlocking (L2/L1), Field (L0) and Onboard - and as a set of typed conduits, each carrying a security profile.

## 4.2 EN 50159 - Safety-related communication and transmission categories

EN 50159 governs safety-related communication over transmission systems and classifies them into three categories. Category 1 (closed) systems have a fixed number of participants, fixed transmission characteristics and no unauthorised access to the medium. Category 2 (controlled) systems are not under full control but the threats are partly mitigated by design. Category 3 (open) systems have an unknown number of participants and an unknown medium, and the possibility of unauthorised access must be assumed. The category dictates the required defences against the canonical EN 50159 threats - repetition, deletion, insertion, re-sequencing, corruption, delay and masquerade - and therefore directly determines what an OT-SOC must monitor: a Category-1 vital bus demands integrity and sequence monitoring; a Category-3 open RF bearer additionally demands timeliness, replay, masquerade and RF-anomaly monitoring.

## 4.3 TS 50701 - Railway cyber security lifecycle

CLC/TS 50701 adapts the IEC 62443 approach to the railway domain and binds it to the EN 5012x safety lifecycle (EN 50126 RAMS, EN 50129 safety case). It requires a documented risk assessment, the allocation of target Security Levels (SL-T) to zones and conduits, and the maintenance of a cyber security case analogous to the safety case. For the OT-SOC, the SL-T per zone is the input that sets monitoring intensity, alert prioritisation and response-time targets; TS 50701 is therefore the standard that makes monitoring an auditable lifecycle obligation and supplies the calibration data for the SOC.

## 4.4 Kavach cyber security requirements

The RDSO Kavach specification and the Kavach Cyber Security SOP impose railway-specific obligations: cryptographic protection of the Kavach radio interface, integrity and replay protection of vital telegrams, controlled key management, and - of direct relevance here - the continuous monitoring and incident-reporting obligations that an OT-SOC discharges. The interlock between the four families is the conceptual core of this paper: IEC 62443 tells the OT-SOC where the boundaries are; EN 50159 tells it what to look for on each conduit; TS 50701 tells it why, to what assurance target and with what evidence; and the Kavach SOP tells it the railway-specific obligations it must satisfy and report against.

# 5. The Architecture-Driven Imperative: Validate Before You Monitor

The central methodological proposition of this paper is that architecture validation must precede OT-SOC deployment. A SOC detects deviation from expected behaviour, and the definition of expected is the architecture. If the architecture is not modelled, the SOC's baseline is whatever traffic happened to exist when monitoring began - including any pre-existing compromise or misconfiguration. In the reference assessment, the architecture was expressed as three separated authorities: a semantic authority (the ontology of asset types, protocols, conduit classes, Purdue levels and protocol capabilities), a governance authority (the zoning, trust boundaries, flow rules, conduit security profiles and EN 50159 transmission categories) and an enforcement authority (the validators that consume the first two and emit findings). This separation is precisely what allows a deviation to be classified unambiguously as a real security gap, a standards-compliance gap, a modelling defect or an enforcement defect.

The assessment demonstrated this concretely. Before validation, several legitimate architectural flows - operator workstation to TMS, the network-management plane, the telecom backhaul, and the Loco-to-Stationary Kavach safety association - were unmodelled, and a vital-to-open-RF flow that should never exist (S-Kavach directly cabled to the radio base station) was present in the topology but indistinguishable from the legitimate ones. Only after the governance model was completed could the forbidden flow be classified as forbidden and prevented, and the legitimate flows be classified as authorised-but-monitored. An OT-SOC fed by the pre-validation model would have had no basis on which to alarm on the forbidden edge. The validated architecture is thus not documentation; it is the configuration source for every sensor, detector and boundary control the OT-SOC will deploy. The residual findings of that assessment - referenced throughout this paper as evidence and consolidated in the residual-risk register of Section 16 - become the OT-SOC's standing monitoring requirements rather than items to be quietly closed.

# 6. OT-SOC Operating Models: Evaluation and Recommendation for Indian Railways

There is no single correct OT-SOC operating model; the right choice depends on scale, geographic footprint, regulatory obligation, existing IT-security capability and the operational maturity of the OT environment. Five archetypes cover the realistic spectrum. Table 2 evaluates each against the specific circumstances of Indian Railways - a national operator with thousands of geographically dispersed wayside and Kavach sites, a tiered zonal/divisional administrative structure, and safety-critical SIL-4 assets.

Table 2. Evaluation of the five OT-SOC operating models for Indian Railways

| Model | Description | Strengths | Suitability for Indian Railways |
| --- | --- | --- | --- |
| A - Dedicated OT-SOC (greenfield) | Physically separate SIEM, analyst workstations and network, OT-native or cross-trained staff, independent stack | Highest detection fidelity; full OT focus | Appropriate at NATIONAL (Tier-3) level under RDSO / Railway Board for the vital estate; high cost justified by SIL-4 criticality |
| B - Fused IT/OT SOC with dedicated OT tier | Shared SIEM/SOAR/ticketing but a ring-fenced OT tier: separate rules, queues, playbooks and at least one OT analyst per shift | Strong capability at manageable cost; reuses existing IT-SOC investment | Appropriate at ZONAL/DIVISIONAL (Tier-2) level; requires a charter giving the OT tier authority to veto IT-SOC actions affecting OT |
| C - MSSP-led remote monitoring | Lean internal team (2-4 FTE) for governance and escalation; day-to-day monitoring outsourced to a specialist OT MSSP | Fast to stand up; compensates for talent scarcity | Transitional only; institutional knowledge must be contractually transferred - unsuitable as the steady state for vital safety |
| D - Asset-owner collaborative (sector-ISAC aligned) | Multiple asset owners contribute telemetry to a sector-level capability; shared L2/L3 threat intelligence | Excellent cross-network threat detection and intelligence | Strongly relevant as a Railway-sector ISAC function, RDSO-anchored and CERT-In / NCIIPC aligned |
| E - Virtual / distributed OT-SOC | Lightweight sensors at each site aggregating to a central analysis platform; analysts across multiple locations | Practical coverage of highly distributed estates | Essential pattern for the thousands of distributed wayside / Kavach / interlocking sites |

Recommendation - a federated hybrid. No single archetype fits a national railway. The recommended model for Indian Railways is a federated hybrid that combines the strengths of four archetypes. A dedicated national OT-SOC (Model A) at Tier-3, operated under RDSO and Railway Board governance, holds the validated architecture, the SL-T allocation, the cyber security case, national threat intelligence and a Railway-sector ISAC function (Model D). Divisional and zonal OT tiers (Model B), fused with existing IT-SOC infrastructure but with explicit authority to override IT actions affecting signalling, provide regional correlation and case management. A distributed sensing layer (Model E) instruments the geographically dispersed wayside, interlocking and Kavach sites with passive collectors that aggregate upward. Model C (MSSP) is admissible only as a time-boxed transitional measure to bridge the talent pipeline, with mandatory knowledge-transfer obligations. Across all tiers, the single non-negotiable governance control is a charter defining decision rights: the OT tier must be able to block any proposed action - including from IT-SOC staff - that would affect a vital system.

# 7. Visibility Architecture: The Five Layers Applied to Railway Assets

Comprehensive OT-SOC visibility requires collecting telemetry from five distinct layers. Many programmes focus exclusively on the network layer and miss the process and identity layers that are most relevant to OT-specific threats. The five layers are: the Network layer (L2/L3 switches, routers, firewalls and IDS sensors); the Asset/Endpoint layer (engineering and operator workstations, jump hosts); the Application/Supervisory layer (TMS, NMS, SOC servers); the Process/Vital-state layer (EI vital state, data loggers, onboard event recorders); and the Identity and Access layer (IDMZ directory services, remote-access gateways, vendor jump servers). Table 3 applies these layers concretely to the railway assets named in the assessment.

Table 3. Five visibility layers applied to Indian Railways OT assets

| Railway asset | Dominant visibility layer(s) | Collection method | Key railway telemetry |
| --- | --- | --- | --- |
| Electronic Interlocking (EI) | Process/Vital-state; Network | Passive SPAN/TAP on EI vital buses; read-only vital-state observation | Route/occupancy state, RaSTA/Kavach-interface reads-writes, safety-code failures, state-machine violations |
| Kavach (S-Kavach / L-Kavach) | Network; Process/Vital-state | Passive TAP plus RF monitor (wayside); onboard event recorder (loco) | Movement-authority frames, MCOMM integrity, sequence/freshness, brake-command integrity, RF anomaly |
| MPLS / PE routers (backbone) | Network | Passive flow monitoring on the telecom core conduit | Transport-security (IPsec/MACsec) tunnel health, routing anomaly, unexpected participants |
| Radio networks (BTS / radio gateway) | Network; Process/Vital-state | RF spectrum monitor plus conduit TAP | Signal-strength and error-rate anomaly (jamming), timeliness/latency deviation, replay indicators |
| Traffic Management System (TMS) | Application/Supervisory | Syslog/TLS forwarding; HTTPS session monitoring | Operator route/setpoint commands, supervisory-command integrity, failed logins, alarm-state changes |
| Network Management System (NMS) | Application/Supervisory; Identity | SNMPv3 event capture; management-plane monitoring | Unexpected SNMP set operations, configuration changes, off-hours management access |
| Data loggers | Process/Vital-state | Read-only log ingestion (never inline) | Forensic event stream; tamper indicators; time-synchronisation drift; gap/reorder detection |
| Engineering workstations (EWS) | Asset/Endpoint; Identity | Endpoint agent where supported, else agentless read-only; PAM session recording | Logic-download and configuration-change actions, USB insertion, anomalous/privileged logins |

The mapping makes two planning points concrete. First, the highest-consequence telemetry - the process/vital-state layer for EI and Kavach - is precisely the telemetry that is hardest to obtain, because vital devices emit few logs and cannot be actively interrogated; it must be derived from passive network observation and from the data loggers and onboard recorders. Second, the identity layer (engineering and vendor access) is the layer most often neglected yet most directly tied to the highest-value attack path, the engineering workstation. A railway OT-SOC that instruments only the network layer will see the backbone but be blind to a logic download in progress on the EI.

# 8. The Monitoring Paradox: Why Railway OT Monitoring Must Remain Passive

Every piece of monitoring infrastructure deployed inside the OT network is itself an attack surface. A compromised sensor or monitoring agent sitting on a vital network is a threat actor's bridge into the most sensitive environment a railway operates. This is the Monitoring Paradox, and in the railway context it has a sharper edge than in any other sector, because the monitored assets are SIL-4 vital systems whose disturbance can directly cause a wrong-side failure.

Three consequences follow. First, monitoring must be passive by default. Active scanning of a vital network - port scans, authenticated polling, vulnerability probes - can reset or destabilise controllers and disrupt time-critical communication; it must require explicit signalling-engineering sign-off for each target and must never be the default mode on the interlocking, field or onboard zones. Telemetry is therefore obtained predominantly through SPAN/TAP and data-diode export. Second, the SOC must remain outside the vital network: the footprint inside the interlocking and onboard zones is minimised, and aggregation and analysis are performed in higher, less-trusted tiers. Third, monitoring must not interfere with the EN 50159 safety layer or the EN 50129 safety case: any device inserted into a vital path could, in principle, introduce delay or corruption, which are themselves EN 50159 threats - so the monitoring apparatus must be demonstrably incapable of affecting the vital communication it observes.

These constraints are not merely operational preferences; they are grounded in the standards. IEC 62443 FR7 (Resource Availability) and the OT priority order (Safety, then Availability, then Integrity, then Confidentiality) forbid security mechanisms that degrade the process. EN 50159's treatment of delay and corruption as first-class threats means that an in-line monitoring device on a SIL-4 bus would be a potential source of the very hazards the safety layer is designed to exclude. The Monitoring Paradox, properly understood, is therefore the railway expression of the principle that in OT the cure must never be more dangerous than the disease.

# 9. Three-Tier OT-SOC Architecture

The federated hybrid recommended in Section 6 is realised as a three-tier architecture organised by geography, latency and function, mirroring both the physical distribution of the railway and the IEC 62443 trust hierarchy. Tier 1 (Station / Wayside) is co-located with the relay or equipment room and comprises passive sensors on the field and interlocking conduits, SPAN/TAP feeds from the EI and object-controller buses, data-logger ingestion and a unidirectional (data-diode) log export toward the upper tiers; it performs local detection and buffers evidence so that connectivity loss does not lose forensic data, and it never hosts an in-line control on a vital path. Tier 2 (Divisional / Zonal OT-SOC) hosts the SIEM, correlation and analytics, intrusion sensors on the IT/OT boundary operating in detection mode toward OT, vulnerability management for non-vital assets and case management; it correlates events across stations in its geography - for example a corridor-wide pattern of balise-telegram anomalies that no single station would detect - and interfaces with divisional signalling control for response. Tier 3 (Central / National OT-SOC) is the custodian of the validated architecture and detection-content repository, the SL-T and cyber security case, cross-zone correlation, national threat intelligence, the Railway-sector ISAC, KPI consolidation and regulatory reporting; when the architecture changes - a new Kavach section, a re-zoned IDMZ - Tier 3 re-validates and pushes updated detection baselines downward. Log flow is strictly upward and, across the IT/OT boundary, diode-enforced; control flow (sensor configuration) is downward and authenticated, preserving the FR5 restricted-data-flow principle within the SOC's own architecture and respecting the Monitoring Paradox.

# 10. Detection Engineering and Railway-Specific Use Cases

Detection engineering converts the validated architecture and its findings into deployable content. The governing principle is that every conduit security requirement and every assessment finding becomes a detection use case: a control that the architecture requires but the traffic may lack is precisely the condition the SOC must detect. The reference assessment surfaced findings that are restated here as durable monitoring requirements. The vital Category-1 buses - EI to S-Kavach over the Kavach EI interface, EI to object controller and speed-sensor to L-Kavach over CAN, and L-Kavach to driver-machine interface and brake-interface unit over MVB - provide integrity and replay protection through safety coding but not cryptographic source authentication. The SOC cannot add authentication, but it can and must detect the symptoms its absence would allow: unexpected source identifiers, protocol state-machine violations and message rates or sequences inconsistent with the vital application. Privileged engineering access is not, in the modelled flows, gated by multi-factor authentication, so every privileged session becomes a high-value detection target. The open RF safety conduits require timeliness, jamming and RF-anomaly monitoring; several trust-boundary crossings are not fully instrumented; and the telecom backhaul carries operational traffic without an evidenced transport-security overlay. Table 4 maps the EN 50159 and IEC 62443 threats to railway detections, and Table 5 expresses the deployable use-case library.

Table 4. Threat-to-detection mapping (EN 50159 / IEC 62443)

| Threat (standard reference) | Railway manifestation | Detection technique | Sensor / tier |
| --- | --- | --- | --- |
| Masquerade / insertion (FR1) | Spoofed axle-counter-evaluator or object-controller message | Source-ID whitelist; state-machine validation; rate anomaly | Interlocking TAP / T1 |
| Corruption (FR3; EN 50159) | Manipulated EI-to-Kavach telegram | Safety-code/CRC failure rate; integrity-flag monitoring | EI/Kavach TAP / T1 |
| Repetition / replay (EN 50159) | Replayed Kavach RF movement-authority frame | Sequence-number regression; freshness deviation | Radio conduit, recorder / T1-T2 |
| Delay / timeliness (FR6; EN 50159) | Stale movement authority over RF | End-to-end latency vs SIL-bounded threshold | Radio conduit / T1-T2 |
| Jamming / availability (FR7) | RF denial against L-Kavach to BTS | RF signal-strength / error-rate anomaly; loss-of-comms | RF monitor / T1 |
| Privileged misuse (FR2) | Unauthorised logic download from EWS | Engineering-session and configuration-change detection; MFA-absence alert | Ops-to-interlocking / T2 |
| Lateral movement (FR5) | Enterprise to operations traversal | Cross-boundary flow correlation; IT-alert time correlation | Trust-boundary sensors / T2 |
| Backhaul tampering (FR3/FR4) | Injection on unprotected MPLS/IP core | Transport-security health; unexpected-participant detection | Telecom conduit / T2 |
| Forbidden topology (FR5) | Direct vital-to-open-RF edge (S-Kavach to BTS) | Topology-conformance vs validated model | Conformance engine / T3 |

Table 5. Railway-specific OT-SOC detection use-case library

| ID | Use case | Trigger | Severity | Safety-aware response |
| --- | --- | --- | --- | --- |
| UC-01 | Spoofed vital field message | Source-ID not in EI whitelist / state-machine violation | SEV1 | Alert control; do not block; manual verification under safe-working rules |
| UC-02 | Kavach EI-interface integrity loss | Rising CRC / safety-code failure | SEV1 | Alert; correlate EI health; engineering investigation |
| UC-03 | Kavach RF replay / delay | Sequence regression / latency beyond SIL threshold | SEV1 | Alert; verify RF environment; no automated RF action |
| UC-04 | RF jamming / loss of comms | Signal-strength collapse / error-rate spike | SEV2 | Alert; invoke degraded-mode working procedures |
| UC-05 | Unauthorised logic download | Logic/config change from EWS toward EI/S-Kavach | SEV1 | Alert; freeze change control; verify authorisation |
| UC-06 | Privileged access without MFA | Jump-host to workstation session, no MFA event | SEV2 | Alert; challenge session; review access policy |
| UC-07 | Trust-boundary bypass | Flow crossing enterprise to operations without required controls | SEV2 | Alert; correlate lateral movement |
| UC-08 | Forbidden topology edge | S-Kavach to BTS (or any forbidden flow) detected | SEV1 | Alert; emergency architecture review |
| UC-09 | Backhaul transport-security failure | Loss/absence of IPsec/MACsec on TMS-MPLS path | SEV3 | Alert; verify tunnel health |
| UC-10 | Balise integrity / position anomaly | Telegram safety-code failure / position implausibility | SEV2 | Alert; NO crypto-encryption alarm (EN 50159 passive-telegram exemption) |
| UC-11 | Data-logger tampering / time drift | Gap, reorder or clock skew in logger stream | SEV3 | Alert; preserve forensic chain |
| UC-12 | Management-plane anomaly | Unexpected SNMPv3 set on NMS to elements | SEV3 | Alert; verify change authorisation |
| UC-13 | New device on vital segment | MAC/IP not in asset inventory on interlocking zone | SEV2 | Alert; verify closed-medium assumption (links to UC-01) |
| UC-14 | Removable media on EWS | USB device-ID not in approved list | SEV3 | Alert; isolate offline EWS per procedure |

A deliberately exempt condition illustrates why the validated architecture matters. The passive balise telegram - the trackside RFID/EUROBALISE air-gap and the reader-to-L-Kavach leg - is protected by the passive safety-coded telegram, with freshness derived from location coding and onboard odometry, not by cryptographic encryption or replay control (EN 50159). The OT-SOC must therefore not raise a missing-encryption or missing-replay alarm on the passive telegram, which would be a false positive (UC-10), while still monitoring telegram integrity and positional plausibility. Encoding standards-justified exemptions as carefully as detections is a discipline that only a validated, authority-separated architecture makes reliable.

Table 6. Trust-boundary monitoring matrix

| Trust boundary | EN 50159 relevance | Required controls | OT-SOC monitoring obligation |
| --- | --- | --- | --- |
| Enterprise (L5) - IDMZ (L3.5) | IT/OT divide | Firewall, inspection | Verify controls present; detect unauthorised ingress; diode log export |
| IDMZ (L3.5) - Operations (L3) | - | Firewall, inspection | Brokered-access audit; jump-host session and MFA-event verification |
| Operations (L3) - Interlocking (L2) | Cat 1 downstream | Firewall, inspection, integrity, authentication | Highest scrutiny; engineering-access and logic-download detection |
| Telecom Core (L2) - Radio Access (L1) | Cat 2 | Firewall, inspection, authentication | Transport-security health; participant verification |
| Radio Access (L1) - Onboard | Cat 3 (open RF) | Integrity, replay, authentication, radio monitoring | Full Cat-3 set: timeliness, jamming, RF anomaly, replay |
| Balise air-gap (Field - Onboard) | Cat 3 passive | Integrity (passive telegram); crypto exempt | Telegram integrity and positional plausibility; no crypto false positives |

# 11. Incident Response Doctrine: The Four-Gate, Safety-First Model

Railway OT incident response inverts the core IT assumption that speed is paramount. IT doctrine is built around speed - contain fast, eradicate fast, recover fast. OT doctrine is built around safety: assess safety impact before every action, contain only when containment does not create a more dangerous state, and recover in coordination with Operations and Engineering. A four-gate model governs every railway OT incident-response action, and no gate permits an action that interrupts a vital path or induces a wrong-side state. The response repertoire is graduated - observe and enrich, then alert the controller, then recommend a manual, procedurally-governed protective action - the last always executed by signalling operations under existing safe-working rules (caution orders, degraded-mode working, temporary speed restrictions), never automatically by the SOC tool.

Table 7. Four-gate OT incident-response doctrine adapted for railway operations

| Gate | Question to answer | Decision authority (railway) |
| --- | --- | --- |
| G1 - Safety assessment | Does the incident, or any proposed response, create or worsen a wrong-side or physical-safety risk? | Signalling / Functional-Safety engineer - mandatory; SOC cannot proceed to containment without a documented safety assessment |
| G2 - Operational-impact assessment | What is the consequence of the incident continuing versus of the proposed response on live train operations? | Divisional Control / Station Master jointly with SOC Tier-3 |
| G3 - Containment decision | What is the minimum viable action that halts threat progression without unacceptable operational impact, and is it reversible? | OT-SOC Lead and Operations Lead - recorded in the incident log |
| G4 - Recovery authorisation | Is the threat remediated, the architecture segment re-validated against the model, and the system safe to return to service? | RDSO / Divisional safety authority - formal return-to-service sign-off |

Decision rights are explicit: the OT-SOC owns detection, enrichment and recommendation; signalling operations owns any action affecting train movement; and RDSO / Railway Board governance owns escalation and external reporting, including the statutory incident notification to CERT-In and the advisories from NCIIPC. Forensic readiness is a precondition, not an afterthought: data loggers and onboard event recorders must have assured integrity and time-synchronisation before an incident, so that evidence is admissible afterward.

# 12. Railway Incident Severity Model (SEV1-SEV4)

Severity in OT must be defined by potential physical consequence, not data value. The following railway-specific scheme anchors severity to the wrong-side failure and to the SIL of the affected asset, and it drives the response SLAs and the escalation chain of Section 14.

Table 8. Railway OT incident severity classification

| Level / name | Definition (consequence-based) | Railway examples | Response SLA and escalation |
| --- | --- | --- | --- |
| SEV1 - SAFETY-CRITICAL | Active threat to physical safety; vital / Kavach / EI compromise confirmed or highly probable; potential wrong-side failure | Manipulation of EI vital logic; spoofed Kavach movement authority; attack on a SIL-4 vital controller | Immediate all-hands; G1 safety assessment invoked; executive and functional-safety notification within 15 min; CERT-In / NCIIPC notification within statutory window |
| SEV2 - OPERATION-CRITICAL | Confirmed OT compromise with capability to disrupt train operations; active lateral movement in OT; containment will affect services | Ransomware on TMS or historian; compromised EWS with live EI/Kavach connection; C2 from an operations server | Tier-3 and SOC-Lead escalation within 15 min; Operations within 30 min; CISO/IRSSE within 1 hour |
| SEV3 - OT-ZONE COMPROMISE | Confirmed malicious presence in OT, not yet demonstrating intent or capability to affect the vital process | Malware on an isolated EWS; phished signalling engineer; rogue device on the OT network | One-hour investigation; Operations informed; CISO notified same business day |
| SEV4 - ANOMALY / POLICY | Policy violation or anomaly with OT scope and no confirmed compromise | Unauthorised removable media; out-of-window vendor access; configuration drift | Next-business-day triage; trend-tracked for tuning and hunting |

# 13. KPI Framework: Board, SOC-Lead and Analyst Tiers

OT-SOC performance must be measured by safety-weighted, standards-anchored indicators rather than enterprise-IT volume metrics; metrics are only valuable if they drive decisions. The framework distinguishes three classes of metric - leading indicators (predictive of future posture), lagging indicators (measuring past performance) and operational-health indicators (measuring SOC sustainability) - and organises them into three reporting tiers, each railway-anchored.

Table 9. Three-tier KPI framework adapted for Indian Railways

| Tier / KPI | Target | Class | Railway meaning and standards anchor |
| --- | --- | --- | --- |
| Board: Vital-asset monitoring coverage | approaching 100% of SIL-3/4 conduits | Leading | Cannot detect what is unseen; FR6, TS 50701 |
| Board: Detection coverage by use case | > 95% active | Leading | Defined use cases with validated, periodically-tested rules |
| Board: Mean Time to Detect (vital) | < 1 hr known TTP; < 4 hr | Lagging | Adversary action to first alert; EN 50129 / FR6 |
| Board: Mean Time to Safe Advisory | minimise (safety-first) | Lagging | Detection to actionable advisory to signalling control; Kavach SOP |
| Board: Incidents contained without operational impact | > 90% | Lagging | Contained without unplanned service stop or safety activation |
| Board: Regulatory-evidence availability | 100% on demand | Op-health | Log retention, access records, incident reports; TS 50701, CERT-In |
| SOC-Lead: Trust-boundary instrumentation | approaching 100% of governance-defined boundaries | Leading | FR5; verify, do not assume |
| SOC-Lead: EN 50159 detector completeness | approaching 100% | Leading | Deployed detectors match each conduit's transmission category |
| SOC-Lead: Sensor / collector uptime | > 99.5% | Op-health | Each gap is a vital blind spot |
| SOC-Lead: Privileged-session audit coverage | 100% | Op-health | Every engineering / vendor session to vital zones reviewed within 24 hr; FR1/FR2 |
| SOC-Lead: New use cases per quarter | > 1 | Leading | Landscape evolves; sourced from intel, post-mortems, red team |
| SOC-Lead: Threat-hunt campaigns per quarter | > 2 | Leading | Only mechanism to find pre-positioned threats |
| Analyst: SPAN/TAP coverage completeness | 100% of L1/L2 vital segments | Op-health | Controller-level visibility is the most important telemetry |
| Analyst: Architecture-conformance drift | approaching 0 | Leading | Observed flows deviating from the validated model; forbidden-flow detections |
| Analyst: False positives on exempt flows | approaching 0 | Op-health | No crypto alarms on the passive balise telegram (EN 50159) |
| Analyst: Detection-content freshness | minimise lag | Op-health | Time since baseline last synchronised to an architecture change |

# 14. Staffing, Competency and Governance Model

The scarcest resource in railway OT security is the individual who is simultaneously fluent in signalling safety and in cyber security. Cross-sector experience confirms a 12-24 month development pipeline and warns that neither shortcut - IT-only analysts, or signalling engineers with no security training - produces effective OT-SOC practitioners. The railway staffing model therefore builds cyber capability atop IRSSE signalling expertise, treating the latter as the harder-to-acquire half, and embeds dedicated railway subject-matter expertise that has no analogue in a generic OT-SOC: the IRSSE signalling SME, the Kavach SME and the Functional Safety specialist.

Table 10. Railway OT-SOC staffing and competency model

| Role | Tier | Core function | Required background |
| --- | --- | --- | --- |
| Tier-1 Analyst (Monitoring Operator) | T1 | Alert triage, initial classification, playbook execution for known scenarios, escalation | Signalling-aware; EI, Kavach, track circuits, axle counters, data loggers; EN 50159 categories; fail-safe principles |
| Tier-2 Analyst (Investigator / Detection Engineer) | T2 | In-depth investigation, network forensics, correlation, detection-content authoring, containment recommendation | Reads railway protocol captures (RaSTA, MCOMM, MVB, CAN); IEC 62443 FRs; liaises with divisional control |
| Tier-3 Analyst (Senior / Threat Hunter) | T3 | Threat hunting, campaign tracking, SIEM tuning, escalation authority for safety-impacting incidents | Deep OT protocol expertise; ICS malware analysis; authority to invoke safety-response procedures |
| IRSSE Signalling SME | T2/T3 | Authoritative interpretation of signalling architecture; conduit and zone context; G2 operational-impact input | IRSSE officer; interlocking and ATP design; SIL allocation; operational railway context |
| Kavach SME | T2/T3 | Kavach RF and EI-interface behaviour; MCOMM and balise telegram semantics; Kavach SOP compliance | Kavach system engineering; RDSO Kavach specification; radio and onboard subsystems |
| Functional Safety Specialist | T3/T4 | Owns the G1 safety-assessment gate; bridges the cyber and safety cases; residual-risk safety judgement | EN 50126 / EN 50129; SIL and safety-case authorship; hazard analysis |
| OT-SOC Lead / IRSSE-CISO interface | T4 | SOC governance, KPI oversight, regulatory reporting, risk-acceptance recommendation, vendor management | Security management (e.g. CISM/CISSP) AND OT context (ISA/IEC 62443 Cybersecurity-Expert level) |

A representative pilot establishment for a single Kavach corridor requires of the order of 14-18 full-time equivalents for continuous 24x7 coverage (Tier-1 to Tier-3 plus a threat hunter and a leave/training reserve), supplemented by the IRSSE, Kavach and functional-safety SMEs who may be shared across the zone. At national scale the Tier-3 dedicated SOC adds threat-intelligence, ISAC and architecture-custody functions. RDSO is the natural custodian of the competency curriculum and the certification scheme that sustains this workforce.

Governance must span the full institutional hierarchy of Indian Railways and the national cyber-security apparatus. Signalling and train-protection systems are Critical Information Infrastructure; their protection therefore engages NCIIPC for designation and national protection oversight and CERT-In for statutory incident reporting and national coordination, in addition to the internal railway hierarchy. Table 8 defines the governance model and the principal responsibility of each entity.

Table 11. National OT-SOC governance and responsibility model

| Entity | Tier / scope | Principal OT-SOC responsibility |
| --- | --- | --- |
| Railway Board | National policy | Policy mandate, funding, national risk-acceptance authority for SIL-4 residual risk, accountability to government |
| RDSO | National technical authority (Tier-3) | Custody of the validated architecture and SL-T; detection-content repository; competency certification; standards and Kavach-SOP interpretation; national OT-SOC operation |
| NCIIPC | National CII protection | Designation of signalling as Critical Information Infrastructure; protection oversight; national-level advisories and audits |
| CERT-In | National CERT | Statutory incident reporting and timelines; national threat intelligence; coordination during national-scale incidents |
| Zonal Railways | Zonal (Tier-2) | Operation of the divisional/zonal fused OT-SOC tier; correlation across the zone; OT veto authority over IT-SOC actions |
| Divisions | Local (Tier-1) | Station and wayside collection; local detection and evidence buffering; first-line response under safe-working rules |

# 15. Residual-Risk Methodology and Risk Acceptance for SIL-4 Systems

Residual risk must be quantified, documented and owned - not buried in assumption logs. The OT-SOC is an assurance instrument, and the open findings of the reference assessment are precisely such residual risk; they are entered into a register with named ownership and a treatment decision, and they remain visible as standing monitoring requirements rather than being silently closed. For SIL-4 systems the methodology imposes additional constraints derived from the EN 50129 safety case and the TS 50701 cyber security case. First, residual risk affecting a SIL-4 vital function cannot be accepted at the SOC or divisional level; it must be escalated to the Railway Board with a supporting safety and cyber-security argument. Second, where a cyber control mandated by IEC 62443 cannot be implemented for sound engineering reasons - the EN 50159 / IEC 62443 authentication tension on vital Category-1 buses is the archetype - the residual risk must be reduced As Low As Reasonably Practicable (ALARP) through compensating controls, of which the OT-SOC's continuous monitoring is itself one, and the residual must be formally argued in the cyber security case. Third, no risk treatment may weaken a SIL-4 boundary, an EN 50159 protection, an authentication or MFA requirement, an integrity or replay-protection requirement, or a trust-boundary control. Table 9 is an extract of the residual-risk register populated from the assessment evidence.

Table 12. Residual-risk register (extract) with SIL-4 acceptance routing

| Ref | Residual risk | Standing monitoring lesson | Treatment / acceptance route |
| --- | --- | --- | --- |
| A-01 | No cryptographic source authentication on vital Cat-1 buses | Police the closed-medium assumption (UC-01/02/13) | Mitigate by monitoring; ALARP; resolve in cyber security case; SIL-4 - Board-level acceptance |
| A-02 | Missing integrity on field-vital train detection (EI to track circuit) | Occupancy-integrity monitoring | Mitigate; SIL-4 - Board-level acceptance |
| A-03 | No MFA on privileged engineering access | Privileged-session monitoring (UC-05/06) | Mitigate now; enforce MFA in architecture; divisional risk owner |
| A-04 | No timeliness / RF-anomaly monitoring on open RF | Category-3 RF instrumentation (UC-03/04) | Mitigate; SIL-4 - Board-level acceptance |
| A-05 | Forbidden vital-to-open-RF edge present in topology | Topology-conformance monitoring (UC-08) | Avoid (remove edge) and detect; architecture authority |
| A-06 | Incompletely instrumented trust boundaries | Boundary verification (UC-07) | Mitigate; OT-SOC and network owners |
| B-01 | EN 50159 / IEC 62443 authentication crediting unresolved | Compensating closure-monitoring | Mitigate by documentation; cyber-safety assurance lead |
| B-02 | TS 50701 SL-T / risk-assessment / cyber-security-case evidence absent | Calibrates all monitoring intensity | Mitigate by producing evidence; RDSO / Railway Board |

# 16. OT-SOC Maturity Model for Indian Railways

A maturity model gives the programme a measurable trajectory. The five-level roadmap below adapts cross-sector maturity practice to the railway context; Indian Railways' realistic near-term target is Level 3 across pilot corridors, progressing to Level 4 nationally as the Tier-3 SOC and the Railway-sector ISAC mature.

Table 13. Five-level OT-SOC maturity roadmap for Indian Railways

| Level | Name | Defining characteristics |
| --- | --- | --- |
| Level 1 | Initial | Ad-hoc passive monitoring at a few sites; no validated architecture; reactive only; reliance on extended IT SOC |
| Level 2 | Defined | Validated architecture and conduit inventory; Tier-1/2 sensors on vital conduits; core use cases (UC-01 to UC-08); SL-T documented |
| Level 3 | Managed | Three-tier OT-SOC operational; four-gate IR doctrine tested against the real process; KPI framework live; EN 50159 category-aware detectors complete |
| Level 4 | Integrated | National Tier-3 SOC and Railway-sector ISAC; proactive threat hunting; cyber security case maintained; architecture-conformance monitoring automated |
| Level 5 | Optimised | Continuous architecture-to-detection synchronisation; leading-indicator-driven; cross-sector intelligence exchange with NCIIPC and CERT-In |

# 17. The 90-Day Pilot-Deployment Programme

To convert strategy into a credible first step, the cross-sector 90-day launch programme is adapted to a realistic Indian Railways pilot on a representative Kavach corridor. The programme assumes that Phase 0 - the production of the validated architecture, the conduit inventory and the TS 50701 SL-T allocation - has been completed, because no sensor should be deployed before the model that configures it exists. The guiding sequencing principle is that breadth never precedes vital depth, and deployment never precedes the validated model.

Table 14. 90-day OT-SOC pilot-deployment programme (Kavach corridor)

| Window | Focus | Key activities |
| --- | --- | --- |
| Days 1-30 | Foundation and assessment | Confirm the validated architecture and SL-T; passive asset discovery on the corridor; ratify the SOC charter and decision rights with the zone; identify SPAN/TAP points on EI, field and boundary conduits; assign initial Tier-1 to Tier-3 staff and SMEs (signalling-first hiring) |
| Days 31-60 | Technology and process | Deploy OT-native network detection with railway-protocol parsing; stand up the OT-contextualised SIEM with asset, zone and criticality context; implement UC-01 to UC-08; draft and table-top the four-gate IR playbooks with divisional Operations and the functional-safety specialist |
| Days 61-90 | Operationalisation and governance | Go live with monitoring; tune false positives (notably the passive-telegram exemption, UC-10); conduct one severity-graded IR exercise against the real process under safe-working rules; publish the first Board and SOC-Lead KPI dashboards; enter the assessment's A and B findings into the residual-risk register and route SIL-4 items to the Board |

# 18. Lessons Learned

The synthesis of railway-specific assessment evidence with field-proven OT-SOC practice yields lessons at two levels. At the technical level, the assessment's findings are durable monitoring requirements: the vital buses rely on a closed-medium assumption that the SOC must continuously police; field-vital integrity must be observed, not assumed; privileged engineering access is the principal cyber pathway to a wrong-side failure and must be monitored as such until MFA is universal; the open RF bearer demands timeliness and RF-anomaly monitoring that cryptography alone does not provide; forbidden topologies must be both prevented and detected; trust boundaries must be instrumented rather than merely declared; the telecom backhaul is in scope; and standards-justified exemptions, such as the passive balise telegram, must be encoded as carefully as detections. At the programme level, an OT-SOC is an operational capability, not a technology purchase - tooling without trained analysts, tested playbooks and decision-rights governance yields visibility without the ability to act, which in an OT environment can be more dangerous than no visibility at all. The mundane and the sophisticated both matter; talent is the binding constraint; the SOC must remain outside the vital network; and residual risk on SIL-4 systems must be owned at Board level, not buried in an assumption log.

# 19. Future Work

Several avenues warrant development. The EN 50159 / IEC 62443 authentication tension on vital Category-1 buses should be resolved in the TS 50701 cyber security case, either through a documented closed-zone argument that credits the safety coding and formally accepts the residual, or through the introduction of cryptographically authenticated vital bearers, with the OT-SOC's compensating monitoring formally credited. Railway-specific detection-content libraries - peer-reviewed signatures and anomaly models for RaSTA, the Kavach interfaces, MVB and balise telegrams - should be curated and shared by RDSO. Machine-assisted architecture-conformance monitoring should continuously diff the validated model against the observed topology to detect drift and forbidden flows automatically, extending UC-08. The safety and security cases should be integrated under TS 50701 and EN 50129 cross-acceptance; the OT-SOC itself should be subjected to resilience engineering so that loss of monitoring does not silently erode assurance; a Railway-sector ISAC should be established and aligned to NCIIPC and CERT-In; and OT-SOC interfaces to the Kavach Cyber Security SOP should be standardised so that detection, escalation and reporting are uniform nationwide.

# 20. Conclusion

The convergence that has modernised Indian Railways signalling - Kavach over open radio, networked electronic interlockings, centralised TMS and NMS over MPLS, and remotely brokered engineering access - has dissolved the isolation on which signalling security historically depended. Protecting this estate requires dedicated Operational Technology Security Operations Centres whose design is fundamentally different from enterprise IT SOCs: passive-first, safety-aware, SIL-prioritised, protocol-literate and bound to a validated architecture. The central contribution of this paper is the argument, evidenced by an independent standards-traceable assessment and reinforced by field-proven OT-SOC practice, that architecture validation must precede OT-SOC deployment, together with the demonstration of how a validated architecture mechanically yields the SOC's operating-model choice, its five-layer visibility plan, its sensor placement, its detection specification, its incident-response and severity models, its KPIs, its staffing and competency needs, its governance responsibilities, its residual-risk register and its maturity roadmap. The recommendation to Indian Railways, RDSO and the Railway Board is unambiguous: invest first in the validated architecture and the TS 50701 evidence base; adopt a federated hybrid SOC model with explicit OT decision rights; instrument the vital conduits and trust boundaries before broadening coverage; build the rare dual-competency workforce on a multi-year horizon under RDSO certification; govern across the Railway Board, RDSO, NCIIPC, CERT-In, the Zonal Railways and the Divisions; and constitute the OT-SOC as a permanent, standards-anchored assurance organ that continuously verifies - through monitoring - the protection on which the safety of the travelling public increasingly depends. Isolation can no longer be assumed; it must now be observed.

# References

[1]  IEC 62443-3-2, Security for industrial automation and control systems - Part 3-2: Security risk assessment for system design, International Electrotechnical Commission.

[2]  IEC 62443-3-3, Security for industrial automation and control systems - Part 3-3: System security requirements and security levels, IEC.

[3]  IEC 62443-2-1, Establishing an industrial automation and control system security programme, IEC.

[4]  IEC 62443-4-2, Technical security requirements for IACS components, IEC.

[5]  CLC/TS 50701, Railway applications - Cybersecurity, CENELEC.

[6]  EN 50159:2010, Railway applications - Communication, signalling and processing systems - Safety-related communication in transmission systems, CENELEC.

[7]  EN 50126, Railway applications - The specification and demonstration of Reliability, Availability, Maintainability and Safety (RAMS), CENELEC.

[8]  EN 50129, Railway applications - Safety-related electronic systems for signalling, CENELEC.

[9]  RDSO, Specification for Train Collision Avoidance System (Kavach), Research Designs and Standards Organisation, Ministry of Railways, Government of India.

[10]  RDSO / Indian Railways, Kavach Cyber Security Standard Operating Procedure (SOP).

[11]  Indian Railways / Railway Board, Signalling Cyber Security Policy and Guidelines.

[12]  NCIIPC, Guidelines for the Protection of Critical Information Infrastructure, National Critical Information Infrastructure Protection Centre, Government of India.

[13]  CERT-In, Directions on information security practices and incident reporting, Indian Computer Emergency Response Team.

[14]  NIST SP 800-82, Guide to Operational Technology (OT) Security, National Institute of Standards and Technology.

[15]  ISA/IEC 62443 Cybersecurity Expert competency programme; ISO/IEC 27001 (enterprise-tier governance reference).

[16]  IRSE, International Technical Committee guidance on cyber security for signalling systems, Institution of Railway Signal Engineers.

[17]  ShieldworkZ, OT SOC Foundational Guide (shieldworkz.com) - cross-sector OT-SOC foundations, operating-model archetypes, five-layer visibility model, the Monitoring Paradox, four-gate incident-response doctrine, severity, KPI, maturity and residual-risk frameworks (primary source adapted and railway-contextualised herein).

[18]  Reference Railway OT Cyber Security Architecture Model and Independent Assessment - separated semantic, governance and enforcement authorities; zone/conduit, EN 50159 transmission-category and SIL traceability (primary evidence base for this paper).

[19]  OT incident references cited for threat context: CRASHOVERRIDE/INDUSTROYER (2016); TRITON/TRISIS (2017); PIPEDREAM/INCONTROLLER (2022); Maroochy Water (2000).